Assignment analysis water quality

Water quality management

Part of the water quality log of the catfish rearing in RAS:

| Catfish (Clarias gariepinus) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Ph | Kh °dKH | Ec m/s | NH4 mg/l | NH3 mg/l | NO3 mg/l | NO2 mg/l | O2 mg/l | Temp ° C |
| 1-2 | 5,5 | 1 | 1 | 10 | 0 | 50 | 5 | 2 | 24 |
| 2-2 | 6,5 | 3 | 2 | 5 | 0 | 75 | 5 | 2 | 27 |
| 3-2 | 7 | 4 | 2 | 2 | 0.01 | 100 | 2 | 2 | 27 |

On 1-Feb the water tests indicated that the water quality at that time was absolutely not up to the requirements.

It was immediately decided to take measures to improve water quality. In order to check the effects, the water values were measured the following days.

- Which parameters indicate the water quality at 1-2 is very poor.

- What could be the underlying cause (s) that cause the water quality to be poor?

- What measures do you think have been taken on 1-2 to improve water quality?

- What measures do you think have been taken on 2-2 to improve water quality?

- What would you suggest to do on 3-2 to further improve water quality?